|  | SI B65 01 02 Sound Systems, Cruise, Alarms, Monitors | March 2002 Service Engineering |
| --- | --- | --- |

SUBJECT
E65 Radio Station "Name" is Displayed Instead of Frequency

MODEL
E65 7 Series

SITUATION
E65 is programmed with RDS "enabled" as a factory setting. As a result some FM radio stations will be displayed as a name instead of the frequency. RDS can be disabled for customers who do not want this feature.

PROCEDURE
Use the following procedure to disable audio system RDS function.

Enter Service Mode of Control Display by:

- In Basic Menu display, press down the Controller for 6 seconds. Hint: The help text flashes briefly when it is ready.

- Turn Controller 3 increments clockwise.

- Turn Controller 3 increments counter clockwise.

- Turn Controller 1 increment clockwise.

- Turn Controller 1 increment counter clockwise.

- Turn Controller 1 increment clockwise.

- Press Controller to confirm.

- Enter "Settings" menu.

- Select "RDS" from menu and you can enable/disable audio system RDS function.

WARRANTY INFORMATION
For Information only.

[ Copyright © 2002 BMW of North America, LLC ]