E60 Springs Installation

FRONTS: (fyi, for removing Hub Bolt: It is a 36mm, 12 point torx bit)

The black plastic cover with a wire coming out of it that comes off the front strut towers is the for the EDC (electronic dampening control unit), which the computer uses to adjust the ride stiffness of the suspension, controlled from inside the car.

Loosen the 3 strut tower nuts (13mm or around there).

On the passenger side is a servo-arm that controls the adaptive headlights. Remove the headlight alignment servo unit, held on by a 10mm nut. Remove these 2 bolts. One is a 16mm bolt that aligns the sway bar bracket, the other that does the same and holds in the bottom of the strut (18mm nut and bolt) so make sure to disconnect before proceeding so you don't snap it while swinging everything down.

Remove Tie rod and rear lower control arm (21mm?)

Jam a chisel to spread open the shock clamp. Shock should now wiggle up and down. I used a little oil just to ease this process.

Next remove all 3 strut bolts. The next step is kind of tough, and I wouldn't recommend it but how I did it was to turn the caliper and rotor in towards the car, put my hand on top of the shock and pushed down hard towards the ground. Next I pulled it out, just missing the fender by mere millimeters.  This way you can remove the spring without removing the entire strut.

Install springs compressor & remove springs.

REARS:

You do not have to remove the seats. You can access from upper sides of trunk. Just remove part of the trunk lining along the sides by removing a few plastic push-pins.  Then access the rear strut towers with the 3 nuts.  EDC attached also back here…

Remove the lower shock bolt 18mm

Remove sway bar links (18mm nut)

Remove upper rear control arm INSIDE link (18mm nut and bolt)

Climb inside your trunk with a push-pin remover, a flashlight, ¼”drive ratchet with short 13mm socket, a MAGNET! and hope you're not claustrophobic!

Peel back carpet lining around the wheel-wells. 2 plastic clips for E60 with fold down seats.

Remove molded foam above the shock

Remove THREE 13mm nuts holding the top of the shock, keep your magnet handy and be VERY careful not to drop these.  They drop behind the frame and its a NIGHTMARE to fish out.

You will hear the shock drop a little ways after the last nut is removed.

CAREFULLY remove the upper rear control arm down and behind where it attaches to give you more room to angle the shock

CAREFULLY slide the bottom of the shock towards the location of where the sway bar attaches. Push down on the brake assembly to give you wiggling room

CAREFULLY push down more and you can slowly push the shock downwards and out.

The shock is ready for removal. Use 13mm and 18mm wrenches to remove top shock plate. No spring compressor is needed.

Reinstall strut into 3 holes at top, without installing top nut yets. This can be frustrating, as you cannot see where the holes line up at the top.

Once aligned, pulled down on brake assembly and slide the bottom of shock to its mounting position. Hand-tighten the 18mm bolt.

Go back inside the trunk, install & tighten down the 3 nuts.

Then go back and tighten down 18mm lower shock bolt, sway bar link and control arm.

_________________________________________

Torque specs:

Front:
Spring strut support bearing: 34 Nm
(3 nuts on the top of the strut tower)

Shock absorber piston rod on thrust bearing: 64 Nm
(nut on the top of the shock)

Spring strut shock absorber to pivot mount: 81 Nm
(nuts that clamp the bottom of the strut)

Rear:
Support on body/rear axel carrier: 28 Nm
(3 nuts on the top of the strut)

Shock absorber piston rod on thrust bearing: 27 Nm
(nut on the top of the shock)

Shock absorber to trailing arm/wheel carrier/swinging arm/support arm/control arm: 165 Nm
(bolt on the bottom of shock)