The black with red stripe and black with brown stripe 20ga wire that goes into the stock sub amp are already a "low level" signal. There's no need to wire them to a line-out converter prior to an amp since the purpose of a line out converter is to convert high-level signals (speaker level output) to line level signals. Solder and splice RCA jacks to these wires and you should be fine.

4.0	RT/GN	pin	10	+ve		[12V+line]
4.0	BR	pin	20	-ve		[12V-lineor earth]
0.35	WS/VI	pin	9	RAD_ON	[Remote on]

0.75	SW/VI	pin	13	AZBL-		[Line level Left input -ve]
0.75	SW/RT	pin	3	AZBL+		[Line level Left input +ve]
0.75	SW/BR	pin	4	AZBR-		[Line level Right input -ve]
0.75	SW/WS	pin	14	AZBR+		[Line level Right input +ve]

2.5	RT/WS	pin	8	ZBL+		[Speaker output Left +ve]
2.5	BR/GE	pin	18	ZBL-		[Speaker output Left -ve]
2.5	RT/BL	pin	6	ZBR+		[Speaker output Right +ve]
2.5 	BR/WS 	pin 	16 	ZBR- 		[Speaker output Right -ve]

WS = White		AZB = Line Level Outputs

RT = Red		ZB = Speaker Connections

GN = Green

BR = Brown

VI = Violet

BL = Blue		2ohms?

SW = Black

GE = Yellow		Low Pass Filter if Whine

GR = Gray