|  |  |  |  | These wheels SHOULD fit E60 XI’s EXACTLY |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Star spoke - style 122 ’07 530xi Std Wheel | 8 x 17 | ET 43 | 245/45R17 | Single wheel Native Chassis | 36116776779 E60 XI |
|  | Star spoke - style 123 07 530xi ETK Optional Wheel | 8JX18 | ET:43 | 245/40 R18W | Single wheel Native Chassis | 36116767540 E60 XI |
|  | Star spoke - style 124 07 530xi ETK Optional Wheel | 8JX18 | ET:43 | 245/40 R18W | Single wheel Native Chassis | 36116775793 E60 XI |
|  | Star spoke - style 138 07 530xi ETK Optional Wheel | 71/2JX17 | ET:43 | 245/45R17 | Single wheel Native Chassis | 36116776777 E60 XI |
|  | Star spoke - style 152 07 530xi ETK Optional Wheel RunFlat Wheel! | 8JX18 | ET:43 | 245/40 R18W | Single wheel Native Chassis | 36117897258 E60 XI |
|  | Star spoke - style 135 | 8JX18 | ET:43 | 245/40 R18W | Single wheel Native Chassis | 36118036944 E60 XI |
|  |  |  |  | These wheels from other BMW’s MAY fit E60 XI’s |  |  |
|  | Double spoke - style 111 | 8 x 17 | ET 46 | 245/45R17 | Single wheel Native Chassis | 36 11 3 401 199 X3 (E83) from 01/2004 |
|  | Double spoke - style 112 | 8 x 17 | ET 46 | 245/45R17 | Single wheel Native Chassis | 36 11 3 401 200 X3 (E83) from 01/2004 |
|  | Star spoke - style 113 | 8 x 18 | ET 46 | 245/40 R18W | Single wheel Native Chassis | 36 11 3 401 201 X3 (E83) from 01/2004 |
|  | Y spoke - style 114 | 8 x 18 Front | ET 46 | 245/40 R18W | Single wheel Native Chassis | 36 11 3 401 202 X3 (E83) from 01/2004 |
|  | Double spoke - style 148 | 8 x 17 | ET 46 | 245/45R17 | Single wheel Native Chassis | 36 11 3 412 060 X3 (E83) from 01/2004 |
|  | M Double spoke - style 135 | 8 x 18 Front | ET 47 | 245/40 R18W | Single wheel Native Chassis | 36 11 7 896 470 3 Series (E46) from 04/98 except M3 Z4 (E85, E86) from 03/2003 |
|  | Cross spoke - style 133 | 8 x 17 | ET 47 | 245/45R17 | Single wheel Native Chassis | 36 11 6 762 109 3 Series (E36) from 12/90 except M3 3 Series (E46) from 04/98 except M3 Z4 (E85, E86) from 03/2003 |
|  | M Double spoke - style 164 | 7,5 x 18 Front | ET 41 | 235/40 R18W | Single wheel Native Chassis | 36 11 2 282 590 3 Series (E46) for M3 and M3 CSL |
|  | Star spoke - style 170 | 8 x 17 | ET 47 | 245/45R17 | Single wheel Native Chassis | 36 11 6 766 740 3 Series (E46) from 04/98 except M3 |
|  | Star spoke - style 171 | 8 x 17 | ET 47 | 245/45R17 | Single wheel Native Chassis | 36 11 6 766 741 3 Series (E46) from 04/98 except M3 |
|  | Turbine styling - style 201 | 8 x 17 | ET 47 | 245/45R17 | Single wheel Native Chassis | 36 11 6 771 158 Z4 (E85, E86) from 03/2003 |
|  | Double spoke - style 205 | 8 x 18 | ET 46 | 235/40 R18W | Single wheel Native Chassis | 36 11 3 417 394 X3 (E83) from 01/2004 |
|  | M Double spoke - style 207 | 7,5 x 17 | ET 47 | 235/45 R17 | Single wheel Native Chassis | 36 11 8 040 048 1 Series (E81, E87) |